VEHICLE DASHBOARD USING CAN PROTOCOL

(.h FILES)

LCD.H

/* lcd.h*/

#include <LPC21xx.H>

#include "delay.h"

#define LCD_D (0xff<<0)

#define RS (1<<8)

#define E  (1<<9)

void LCD_INIT(void);

void LCD_COMMAND(unsigned char);

void LCD_DATA(unsigned char);

void LCD_INTEGER(int);

void LCD_STR(unsigned char *);

void LCD_SCROLL(unsigned char *);

void LCD_FLOAT(float);

void LCD_INIT(void)

{

IODIR0 |= LCD_D|RS|E;

LCD_COMMAND(0x01);

LCD_COMMAND(0x02);

LCD_COMMAND(0x0C);

LCD_COMMAND(0x38);

}

void LCD_COMMAND(unsigned char cmd)

{

IOCLR0 = LCD_D;

IOSET0 = cmd;

IOCLR0 = RS;

IOSET0 = E;

delay_millisecond(2);

IOCLR0 = E;

}

void LCD_DATA(unsigned char d)

{

IOCLR0 = LCD_D;

IOSET0 = d;

IOSET0 = RS;

IOSET0 = E;

delay_millisecond(2);

IOCLR0 = E;

}

void LCD_INTEGER(int n)

{

unsigned char num[20];

int i=0;

if(n==0)

{

LCD_DATA('0');

return;

}

if(n<0)

{

LCD_DATA('-');

n=-n;

}

while(n>0)

{

num[i++]=n%10;

n=n/10;

}

for(i=i-1;i>=0;i--)

{

LCD_DATA(num[i]+48);

}

}

void LCD_FLOAT(float n)

{

int temp=(int)n;

LCD_INTEGER(temp);

LCD_DATA('.');

LCD_INTEGER((n-temp)*100);

}

void LCD_STR(unsigned char *d)

{

while(*d!='\0')

{

LCD_DATA(*d++);

}

}

void LCD_SCROLL(unsigned char *s)

{

int len=0,i,j;

while(s[len]!='\0')

len++;

for(i=0;i<=len;i++)

{

LCD_COMMAND(0x80);

for(j=0;j<16;j++)

{

if((i+j)<len)

LCD_DATA(s[i+j]);

else

LCD_DATA(' ');

}

delay_millisecond(300);

}

}

DELAY.H

/*delay.h*/

#include <LPC21xx.H>

void delay_millisecond(unsigned int millisecond)

{

T0PR=15000-1;

T0TCR=0x01;

while(T0TC<millisecond);

T0TCR=0x03;

T0TCR=0x00;

}

void delay_second(unsigned int second)

{

T0PR=15000000-1;

T0TCR=0x01;

while(T0TC<second);

T0TCR=0x03;

T0TCR=0x00;

}

void delay_microsecond(unsigned int microsecond)

{

T0PR=15-1;

T0TCR=0x01;

while(T0TC<microsecond);

T0TCR=0x03;

T0TCR=0x00;

}

CAN.H

/* can_d.h */

#include <LPC21xx.H>

typedef struct CAN2

{

unsigned int id;

unsigned int rtr;

unsigned int dlc;

unsigned int byteA;

unsigned int byteB;

}CAN2_MSG;

/* function prototypes */

void can2_init(void);

void can2_tx(CAN2_MSG m1);

void can2_rx(CAN2_MSG *m1);

/* ---------------- CAN INIT ---------------- */

void can2_init(void)

{

PINSEL1 |= 0x00014000;     /* P0.23 = RD2 , P0.24 = TD2 */

VPBDIV = 1;               /* PCLK = CCLK */

C2MOD = 0x01;            /* reset mode */

AFMR  = 0x02;            /* accept all */

C2BTR = 0x001C001D;      /* 125 kbps */

C2MOD = 0x00;            /* normal mode */

}

/* ---------------- CAN TX ---------------- */

void can2_tx(CAN2_MSG m1)

{

C2TID1 = m1.id;

C2TFI1 = (m1.dlc << 16);

if(m1.rtr == 0)

{

C2TFI1 &= ~(1<<30);   /* data frame */

C2TDA1 = m1.byteA;

C2TDB1 = m1.byteB;

}

else

{

C2TFI1 |= (1<<30);    /* remote frame */

}

C2CMR = (1<<0) | (1<<5);

while((C2GSR & (1<<3)) == 0);

}

/* ---------------- CAN RX ---------------- */

void can2_rx(CAN2_MSG *m1)

{

while((C2GSR & 1) == 0);

m1->id  = C2RID;

m1->dlc = (C2RFS >> 16) & 0x0F;

m1->rtr = (C2RFS >> 30) & 0x01;

if(m1->rtr == 0)

{

m1->byteA = C2RDA;

m1->byteB = C2RDB;

}

C2CMR = (1<<2);          /* release buffer */

}