VEHICLE DASHBOARD USING CAN PROTOCOL

DASHBOARD TRANSMITTER CODE

/* ========================= transmit.c ========================= */

/* Main Dashboard Node */

#include <LPC21xx.H>

#include "lcddisplay.h"

#include "hadd.h"

#define SW1 10

#define SW2 11

#define M1 16

#define M2 17

#define LED 19          /* active low */

int main()

{

CAN2_MSG m1,m2;

unsigned int v;

LCD_INIT();

can2_init();

/* OUTPUT PINS */

IODIR0 |= (1<<M1) | (1<<M2) | (1<<LED);

/* SWITCH INPUT PINS */

IODIR0 &= ~(1<<SW1);

IODIR0 &= ~(1<<SW2);

/* INITIAL STATES */

IOCLR0 = (1<<M1) | (1<<M2);

IOSET0 = (1<<LED);

LCD_COMMAND(0x80);

LCD_STR("VEHICLE");

LCD_COMMAND(0xC0);

LCD_STR("DASHBOARD");

delay_second(2);

LCD_COMMAND(0x01);

while(1)

{

/* ---------- TEMPERATURE ---------- */

if(((IOPIN0>>SW1)&1)==0)

{

delay_millisecond(200);

m1.id    = 0x101;

m1.rtr   = 1;

m1.dlc   = 1;

m1.byteA = 0;

m1.byteB = 0;

can2_tx(m1);

can2_rx(&m2);

if((m2.id==0x101) && (m2.rtr==0))

{

LCD_COMMAND(0x01);

LCD_COMMAND(0x80);

LCD_STR("TEMP:");

LCD_INTEGER(m2.byteA);

LCD_DATA(0xDF);

LCD_DATA('C');

if(m2.byteA > 25)

{

IOSET0 = (1<<M1);

IOCLR0 = (1<<M2);

}

else

{

IOCLR0 = (1<<M1);

IOCLR0 = (1<<M2);

}

}

while(((IOPIN0>>SW1)&1)==0);

}

/* ---------- FUEL ---------- */

if(((IOPIN0>>SW2)&1)==0)

{

delay_millisecond(200);

m1.id    = 0x102;

m1.rtr   = 1;

m1.dlc   = 1;

m1.byteA = 0;

m1.byteB = 0;

can2_tx(m1);

can2_rx(&m2);

if((m2.id==0x102) && (m2.rtr==0))

{

v = m2.byteA;

LCD_COMMAND(0x01);

LCD_COMMAND(0x80);

if(v < 10)

{

LCD_STR("FUEL:LOW");

IOCLR0 = (1<<LED);

}

else if(v < 25)

{

LCD_STR("FUEL:MEDIUM");

IOSET0 = (1<<LED);

}

else

{

LCD_STR("FUEL:HIGH");

IOSET0 = (1<<LED);

}

LCD_COMMAND(0xC0);

LCD_STR("VOLT:");

LCD_INTEGER(v/10);

LCD_DATA('.');

LCD_INTEGER(v%10);

LCD_DATA('V');

}

while(((IOPIN0>>SW2)&1)==0);

}

}

}

TEMPERATURE RECEIVER CODE

/* ========================= receiver.c ========================= */

/* Temperature Node */

#include <LPC21xx.H>

#include "can_d.h"

#define CS        7

#define MSTR_BIT  5

#define Mode_3    0x18

#define SPIF_BIT  7

CAN2_MSG m1,m2;

unsigned char SPI0(unsigned char data);

float Read_ADC_MCP3204(unsigned char ch);

/* SPI INIT */

void Init_SPI0(void)

{

PINSEL0 |= 0x00001500;

S0SPCCR = 60;

S0SPCR  = (1<<MSTR_BIT) | Mode_3;

IODIR0 |= (1<<CS);

IOSET0  = (1<<CS);

}

/* SPI DATA */

unsigned char SPI0(unsigned char data)

{

unsigned char stat;

stat = S0SPSR;

S0SPDR = data;

while((S0SPSR & (1<<SPIF_BIT)) == 0);

return S0SPDR;

}

/* ADC READ */

float Read_ADC_MCP3204(unsigned char ch)

{

unsigned char h,l;

unsigned int val = 0;

IOCLR0 = (1<<CS);

SPI0(0x06);

h = SPI0(ch<<6);

l = SPI0(0x00);

IOSET0 = (1<<CS);

val = ((h & 0x0F)<<8) | l;

return ((val * 3.3) / 4096);

}

/* MAIN */

int main()

{

float volt;

int temp;

can2_init();

Init_SPI0();

while(1)

{

volt = Read_ADC_MCP3204(0);

temp = volt * 100;

can2_rx(&m1);

if((m1.id==0x101) && (m1.rtr==1))

{

m2.id    = 0x101;

m2.rtr   = 0;

m2.dlc   = 1;

m2.byteA = temp;

m2.byteB = 0;

can2_tx(m2);

}

}

}

FUEL RECEIVER CODE

/* ---------- receiver2.c ---------- */

/* Fuel Node */

#include <LPC21xx.H>

#include "can_d.h"

#define CS        7

#define MSTR_BIT  5

#define Mode_3    0x18

#define SPIF_BIT  7

CAN2_MSG m1,m2;

unsigned char SPI0(unsigned char data);

float Read_ADC_MCP3204(unsigned char ch);

/* SPI INIT */

void Init_SPI0(void)

{

PINSEL0 |= 0x00001500;

S0SPCCR = 60;

S0SPCR  = (1<<MSTR_BIT) | Mode_3;

IODIR0 |= (1<<CS);

IOSET0  = (1<<CS);

}

/* SPI DATA */

unsigned char SPI0(unsigned char data)

{

unsigned char stat;

stat = S0SPSR;

S0SPDR = data;

while((S0SPSR & (1<<SPIF_BIT)) == 0);

return S0SPDR;

}

/* ADC READ */

float Read_ADC_MCP3204(unsigned char ch)

{

unsigned char h,l;

unsigned int val = 0;

IOCLR0 = (1<<CS);

SPI0(0x06);

h = SPI0(ch<<6);

l = SPI0(0x00);

IOSET0 = (1<<CS);

val = ((h & 0x0F)<<8) | l;

return ((val * 3.3) / 4096.0);

}

/* MAIN */

int main()

{

float volt;

can2_init();

Init_SPI0();

while(1)

{

volt = Read_ADC_MCP3204(0);

can2_rx(&m1);

if((m1.id==0x102) && (m1.rtr==1))

{

m2.id    = 0x102;

m2.rtr   = 0;

m2.dlc   = 1;

m2.byteA = (unsigned int)(volt * 10);

m2.byteB = 0;

can2_tx(m2);

}

}

}